ANNEXE B:

PROGRAMME ASSEMBLEUR  DE CALCUL LA FFT A  ENTERLACEMENT TEMPOREL RADIX-2

MODULE PRINCIPAL

INSTR_B       .set   0f073h

ADDR_INTR_3   .set   020Ch

fft_data      .set     2200h

d_input_addr  .set   2300h

d_output_addr .set   2200h

initst0	      .set   1800h	;set st0 initial number

initst1	      .set   2a40h	;set st1 initial number

DEBUG         .set   0208h      ; breakpoint srv enter address !

K_ST1	      .set   2a40h	; st1 reset value

K_FFT_SIZE    .set   128	; 128 point complex fft

K_LOGN	      .set   7          ; # of stages (=logN/log2)

K_ZERO_BK     .set   0

K_DATA_IDX_1  .set   2  	; Data index for Stage 1

K_DATA_IDX_2  .set   4  	; Data index for Stage 2

K_DATA_IDX_3  .set   8  	; Data index for Stage 3

K_TWID_TBL_SIZE .set 128 	; Twiddle table size

K_FLY_COUNT_3 .set   4    	; Butterfly counter for Stage 3

K_TWID_IDX_3  .set   32    	; Twiddle index for Stage 3

/*Coefficients du filtre TFR*/

.data

sine_table   .word   0h

.word   324h

.word   647h

.word   96ah

.word   0c8bh

.word   0fabh

.word   12c7h

.word   15e1h

.word   18f8h

.word   1c0bh

.word   1f19h

.word   2223h

.word   2527h

.word   2826h

.word   2b1eh

.word   2e10h

…

.data

cos_table    .word   7fffh

.word   7ff5h

.word   7fd7h

.word   7fa6h

.word   7f61h

.word   7f08h

.word   7e9ch

.word   7e1ch

.word   7d89h

.word   7ce2h

.word   7c29h

.word   7b5ch

.word   7a7ch

.word   7989h

.word   7883h

.bss temp_flag,1

.bss test_val,1

.bss temp_ar5,1

.bss temp_ar3,1

.bss input_cnt,1                           ;input counter

.bss output_cnt,1                           ;output counter

.bss d_grps_cnt,1                          ;group counter

.bss d_twid_idx,1                          ;index of twiddle table

.bss d_data_idx,1                          ;index of input data

.bss stack_buffer,100h

.mmregs

.global _c_int00

.asg AR1,FFT_TWID_P

.asg AR2,REORDERED_DATA                ;pointer to processed data in bit_reversed order

.asg AR3,ORIGINAL_INPUT                  ;pointer to original input data in natural order

.asg ar4,d_input_backup

.asg ar5,OUTPUT_BUF

.asg AR7,DATA_PROC_BUF                   ;starting addressing of data processing buffer

.text

_c_int00:

stm #initst0,st0					;initialize st0

stm #initst1,st1					;initialize st1

stm #stack_buffer+100h,sp

ld #temp_flag,dp

ld #0,a

st #0,input_cnt

st #d_input_addr,temp_ar3	  	; store 1 st original input

wait_input:

rsbx intm                       ;enable all int

nop

call get_input                  ; set toggle point in CCS !

; read FFT source data to 0x2300 (256 words)!

nop                             ; set breakpoint in CCS !

process:

ssbx intm

CALL bit_rev

CALL fft

CALL unpack

CALLD power

STM #K_ST1,ST1

stm #d_output_addr,OUTPUT_BUF

b _c_int00         ; return to begin

nop

nop

nop

get_input:

nop                   ;a->AD data input to 0x2300:256

RET

/*………………………………………………………………………….*/

{……LA ROUTINE D’INVERSION DES BITS……}

bit_rev:

pshm st0

pshm ar0

pshm bk

SSBX FRCT                                ; fractional mode is on;

SSBX INTM

STM #d_input_addr,ORIGINAL_INPUT         ;AR3->1 st original input

STM #fft_data,DATA_PROC_BUF              ;AR7-> data processing buffer

MVMM DATA_PROC_BUF,REORDERED_DATA        ;AR2->1st bit_reversed data

STM #K_FFT_SIZE-1,BRC

STM #K_FFT_SIZE,AR0                      ;AR0 = 1/2 size of circ buffer

RPTB bit_rev_end-4

MVDD *ORIGINAL_INPUT+,*REORDERED_DATA+

MVDD *ORIGINAL_INPUT-,*REORDERED_DATA+

MAR *ORIGINAL_INPUT+0B

popm bk

popm ar0

popm st0

bit_rev_end:

RET

{…calcul FFT radix2 à entrelacement temporel….}

fft:

; --------- stage1 ------------------------

.asg AR1,GROUP_COUNTER

.asg AR2,PX

.asg AR3,QX

.asg AR4,WR

.asg AR5,WI

.asg AR6,BUTTERFLY_COUNTER

.asg AR7,DATA_PROC_BUF                    ; for Stages 1 & 2

.asg AR7,STAGE_COUNTER                    ; for the remaining stages

pshm st0

pshm ar0

pshm bk

SSBX SXM

STM #K_ZERO_BK,BK                         ;BK=0 so that *ARn+0% == *ARn+0

LD #-1,ASM                                ;outputs div by 2 at each stage

MVMM DATA_PROC_BUF,PX                     ;PX->PR

LD *PX,16,A                                  ;A := PR

STM #fft_data+K_DATA_IDX_1,QX             ;QX->QR

STM #K_FFT_SIZE/2-1,BRC

RPTBD stage1end-1

STM #K_DATA_IDX_1+1,AR0			; two byte instruction

SUB *QX,16,A,B                            ; B := PR-QR

ADD *QX,16,A                              ; A := PR+QR

STH A,ASM,*PX+                            ; PR':= (PR+QR)/2

ST B,*QX+                                 ; QR':= (PR-QR)/2

||LD *PX,A                                ; A := PI

SUB *QX,16,A,B                            ; B := PI-QI

ADD *QX,16,A                              ; A := PI+QI

STH A,ASM,*PX+0%                           ; PI':= (PI+QI)/2

ST B,*QX+0%                               ; QI':= (PI-QI)/2

||LD *PX,A                                ; A := next PR

stage1end:

;…………. Stage 2 ………………………..

MVMM DATA_PROC_BUF,PX                     ; PX ? PR

STM #fft_data+K_DATA_IDX_2,QX             ; QX ? QR

STM #K_FFT_SIZE/4-1,BRC

LD *PX,16,A                                  ; A := PR

RPTBD stage2end-1

STM #K_DATA_IDX_2+1,AR0

; 1st butterfly

SUB *QX,16,A,B                            ; B := PR-QR

ADD *QX,16,A                              ; A := PR+QR

STH A,ASM,*PX+                            ; PR':= (PR+QR)/2

ST B,*QX+                                 ; QR':= (PR-QR)/2

||LD *PX,A                                ; A := PI

SUB *QX,16,A,B                            ; B := PI-QI

ADD *QX,16,A                              ; A := PI+QI

STH A,ASM,*PX+                            ; PI':= (PI+QI)/2

STH B,ASM,*QX+                            ; QI':= (PI-QI)/2

; 2nd butterfly

MAR *QX+

ADD *PX,*QX,A                             ; A := PR+QI

SUB *PX,*QX-,B                            ; B := PR-QI

STH A,ASM,*PX+                            ; PR':= (PR+QI)/2

SUB *PX,*QX,A                             ; A := PI-QR

ST B,*QX                                  ; QR':= (PR-QI)/2

||LD *QX+,B                               ; B := QR

ST A, *PX                                 ; PI':= (PI-QR)/2

||ADD *PX+0%,A                            ; A := PI+QR

ST A,*QX+0%                               ; QI':= (PI+QR)/2

||LD *PX,A                                ; A := PR

stage2end:

; ………..Stage 3 thru Stage logN-1………..

STM #K_TWID_TBL_SIZE,BK                   ; BK = twiddle table size always

ST #K_TWID_IDX_3,d_twid_idx               ; init index of twiddle table

STM #K_TWID_IDX_3,AR0                     ; AR0 = index of twiddle table

STM #cos_table,WR                      ; init WR pointer

STM #sine_table,WI                        ; init WI pointer

STM #K_LOGN-2-1,STAGE_COUNTER             ; init stage counter

ST #K_FFT_SIZE/8-1,d_grps_cnt             ; init group counter

STM #K_FLY_COUNT_3-1,BUTTERFLY_COUNTER    ; init butterfly counter

ST #K_DATA_IDX_3,d_data_idx               ; init index for input data

stage:

STM #fft_data,PX 				; PX->PR

LD d_data_idx, A

ADD *(PX),A

STLM A,QX                                 ; QX ? QR

MVDK d_grps_cnt,GROUP_COUNTER             ; AR1 contains group countergroup:

group:

MVMD BUTTERFLY_COUNTER,BRC                ; # of butterflies in each grpTER,BRC

RPTBD butterflyend-1

LD *WR,T

MPY *QX+,A                                ; A := QR*WR || QX?QI

MAC *WI+0%,*QX-,A                        ; A := QR*WR+QI*WI

ADD *PX,16,A,B                               ; B := (QR*WR+QI*WI)+PR

ST B,*PX                                  ; PR':=((QR*WR+QI*WI)+PR)/2

||SUB *PX+,B                              ; B := PR-(QR*WR+QI*WI)

ST B,*QX                                  ; QR':= (PR-(QR*WR+QI*WI))/2

||MPY *QX+,A                              ; A := QR*WI [T=WI]

MAS *QX,*WR+0%,A                         ; A := QR*WI-QI*WR

ADD *PX,16,A,B                            ; B := (QR*WI-QI*WR)+PI

ST B,*QX+                                 ; QI':=((QR*WI-QI*WR)+PI)/2

||SUB *PX,B                               ; B := PI-(QR*WI-QI*WR)

LD *WR,T                                  ; T := WR

ST B,*PX+                                 ; PI':= (PI-(QR*WI-QI*WR))/2

||MPY *QX+,A                              ; A := QR*WR || QX?QI

butterflyend:

; Update pointers for next group

PSHM AR0                                ; preserve AR0

MVDK d_data_idx,AR0

MAR *PX+0                               ; increment PX for next group

MAR *QX+0                               ; increment QX for next group

BANZD group,*GROUP_COUNTER-

POPM AR0                                ; restore AR0

MAR *QX- ;	trap 2

; Update counters and indices for next stage

LD d_data_idx,A

SUB #1,A,B                              ; B = A-1

STLM B,BUTTERFLY_COUNTER                ; BUTTERFLY_COUNTER = #flies-1

STL A,1,d_data_idx                      ; double the index of data

LD d_grps_cnt,A

STL A,-1,d_grps_cnt                    ; 1/2 the offset to next group

LD d_twid_idx,A

STL A,-1,d_twid_idx                    ; 1/2 the index of twiddle table

BANZD stage,*STAGE_COUNTER-

MVDK d_twid_idx,AR0                     ; AR0 = index of twiddle table

popm bk

popm ar0

popm st0

fft_end:

RET

/*………………………………………………………………..*/

Unpack:

.asg AR2,XP_k

.asg AR3,XP_Nminusk

.asg AR6,XM_k

.asg AR7,XM_Nminusk

pshm st0

pshm ar0

pshm bk

SSBX INTM

SSBX FRCT

SSBX SXM

LD #-1,ASM

STM #fft_data+2,XP_k                    ; AR2 -> R[k] (temp RP[k])

STM #fft_data+2*K_FFT_SIZE-2,XP_Nminusk ; AR3 -> R[N-K] (temp RP[N-K])

STM #fft_data+2*K_FFT_SIZE+3,XM_Nminusk ; AR7 -> temp RM[N-K]

STM #fft_data+4*K_FFT_SIZE-1,XM_k       ; AR6 -> temp RM[K]

STM #-2+K_FFT_SIZE/2,BRC

RPTBD phase3end-1

STM #3,AR0

ADD *XP_k,*XP_Nminusk,A                 ; A := R[k]+R[N-K] = 2*RP[k]

SUB *XP_k,*XP_Nminusk,B                 ; B := R[k]-R[N-K]= 2*RM[k]

STH A,ASM,*XP_k+                        ; store RP[k] at AR[k]

STH A,ASM,*XP_Nminusk+                  ; store RP[N-K]=RP[k] at AR[N-K]

STH B,ASM,*XM_k-                        ; store RM[k] at AI[2N-K]

NEG B                                   ; B := R[N-K]-R[k] =2*RM[N-K]

STH B,ASM,*XM_Nminusk-                  ; store RM[N-K] at AI[N+k]

ADD *XP_k,*XP_Nminusk,A                 ; A := I[k]+I[N-K] = 2*IP[k]

SUB *XP_k,*XP_Nminusk,B                 ; B := I[k]-I[N-K] =2*IM[k]

STH A,ASM,*XP_k+                        ; store IP[k] at AI[k]

STH A,ASM,*XP_Nminusk-0                 ; store IP[N-K]=IP[k] at AI[N-K]

STH B,ASM,*XM_k-                        ; store IM[k] at AR[2N-K]

NEG B                                   ; B := I[N-K]-I[k] =2*IM[N-K]

STH B,ASM,*XM_Nminusk+0                 ; store IM[N-K] at AR[N+k]

phase3end:

ST #0,*XM_k-                            ; RM[N/2]=0

ST #0,*XM_k                             ; IM[N/2]=0

; Compute AR[0],AI[0], AR[N], AI[N]

.asg AR2,AX_k

.asg AR4,IP_0

.asg AR5,AX_N

STM #fft_data,AX_k                       ; AR2 -> AR[0] (temp RP[0])

STM #fft_data+1,IP_0                     ; AR4 -> AI[0] (temp IP[0])

STM #fft_data+2*K_FFT_SIZE+1,AX_N        ; AR5 -> AI[N]

ADD *AX_k,*IP_0,A                        ; A := RP[0]+IP[0]

SUB *AX_k,*IP_0,B                        ; B := RP[0]-IP[0]

STH A,ASM,*AX_k+                         ; AR[0] = (RP[0]+IP[0])/2

ST #0,*AX_k                              ; AI[0] = 0

MVDD *AX_k+,*AX_N-                       ; AI[N] = 0

STH B,ASM,*AX_N                          ; AR[N] = (RP[0]-IP[0])/2

; Compute final output values AR[k], AI[k]

.asg AR3,AX_2Nminusk

.asg AR4,COS

.asg AR5,SIN

STM #fft_data+4*K_FFT_SIZE-1,AX_2Nminusk 	; AR3 -> AI[2N-1](temp RM[1])

STM #cos_table+K_TWID_TBL_SIZE/K_FFT_SIZE,COS 	; AR4 -> cos(k*pi/N)

STM #sine_table+K_TWID_TBL_SIZE/K_FFT_SIZE,SIN	; AR5 ->  sin(k*pi/N)

STM #K_FFT_SIZE-2,BRC

RPTBD phase4end-4

STM #K_TWID_TBL_SIZE/K_FFT_SIZE,AR0      ; index of twiddle tables

LD *AX_k+,16,A                           ; A := RP[k] || AR2->IP[k]

MACR *COS,*AX_k,A                        ; A :=A+cos(k*pi/N)*IP[k]

MASR *SIN,*AX_2Nminusk-,A                ; A := A-sin(k*pi/N)*RM[k] || AR3->IM[k]

LD *AX_2Nminusk+,16,B                    ; B := IM[k] ||AR3->RM[k]

MASR *SIN+0%,*AX_k-,B                    ; B := B-sin(k*pi/N)*IP[k] || AR2->RP[k]

MASR *COS+0%,*AX_2Nminusk,B              ; B := B-cos(k*pi/N)*RM[k]

STH A,ASM,*AX_k+                         ; AR[k] = A/2

STH B,ASM,*AX_k+                         ; AI[k] = B/2

NEG B                                    ; B := -B

STH B,ASM,*AX_2Nminusk-                  ; AI[2N-K] = -AI[k]= B/2

STH A,ASM,*AX_2Nminusk-                  ; AR[2N-K] = AR

; [k] = A/2

popm bk

popm ar0

popm st0

phase4end:

;	intr 2

RET

/*…………………………………………………………………………….*/

power:

.asg AR2,AX

.asg AR3,OUTPUT_BUF

pshm st0

pshm ar0

pshm bk

;	trap 2

SSBX INTM

SSBX FRCT

SSBX SXM

STM #d_output_addr,OUTPUT_BUF              ; AR3 points to output buffer

STM #K_FFT_SIZE*2-1,BRC

RPTBD power_end-4

STM #fft_data,AX                           ; AR2 points to AR[0]

SQUR *AX+,A                                ; A := AR^2

SQURA *AX+,A                               ; A := AR^2 + AI^2

;        trap 2

STH A,7,*OUTPUT_BUF

ANDM #7FFFH,*OUTPUT_BUF+ ;        intr 2

popm bk

popm ar0

popm st0

power_end:

RET                                      ; return to main program

CONFIGURATION DE  L’ENVIRONNEMENT DE DSP TMS320C541

MEMORY

{

PAGE 0:

INT_PM_DRAM:	origin = 80h	length = 1380h	  /* DARAM if ovly=1 */

EXT_PM_RAM:	origin = 1400h	length = 0ec00h   /* ext PM 0-wait */

PAGE 1:

SCRATCH:	origin = 00060h  length = 00020h  /* page-0 scratch-pad */

INT_DM_RAM:	origin = 80h	 length = 1380h   /* DARAM */

EXT_DM_RAM:	origin = 01400h  length = 0ec00h  /* ext DM 0-wait */

}

SECTIONS

{

.text   :	 > EXT_PM_RAM	PAGE 0

.cinit  :	 > EXT_PM_RAM	PAGE 0

.sintab :	 > INT_DM_RAM	PAGE 1

.input  :	 > INT_DM_RAM	PAGE 1,align(2048)

.bss    :	 > INT_DM_RAM	PAGE 1

.stack  :	 > INT_DM_RAM	PAGE 1

.xref   :	 > EXT_DM_RAM	PAGE 1

.test   :	 > EXT_DM_RAM	PAGE 1

}