CHAPTER 3

PROPOSED WORK

Introduction

This part is the core of our work. The mobility of nodes is the main factor which prompted us to propose a link failure prediction method and thinking to the need of improving the routing performance in MANET environments, by avoiding the frequent link breakage. In This chapter, section 3.2 will present the problem statement, followed by the proposed solution in section 3.3 which we suggested for getting better performance.

Problem identification

As mentioned in chapter 1, in the standard AODV protocol, when a link break occurs, the path has to be repaired (local recovery) or a new path has to be discovered. During alternate route discovery after link break, packets will be dropped. This leads to wastage of the scarce node resources and decreases the routing performance in terms of many metrics such as end to end delay and packet delivery ratio.

The next section (section 3.3) will give a solution for this problems, so, how can we improve the network performance?.

Proposed solution

In this section, we propose a link failure prediction algorithm AODV-LP (AODV with Link Prediction) Based on the Ad hoc On-demand Distance Vector AODV routing protocol, uses the received signal strength, RSS, and the received signal strength changing rate, to predict the time changing rate after which an active link will break. This is done by estimating the time at which received signal strength of the Hello packets will fall below a threshold power. The received power level below the threshold indicates that the two nodes are moving away from each other’s radio transmission range. The nodes estimate the link breakage time and discover another route to the destination much earlier.

In this approach, two consecutive measurements of signal strength of packets received from the neighboring node are used to predict the link failure using the Newton divided difference method (The Newton interpolation polynomial) [48].

The received signal strengths of the tow latest Hello packets and their time of occurrence are maintained by each receiver for each transmitter from which it is receiving. Using tow received Hello packets signal power strengths as P1, P2 and the time when Hello packets arrived as T1, T2 instants respectively and Pthreshold as the threshold signal strength to be operative at the time Tthreshold, one can predict Tthreshold. We assume that at the predicted time Tthreshold, when received power level reduces to threshold power, the link will break. The threshold signal strength Pthreshold, is the minimum power receivable by the device. This is the power at the maximum transmission range.

The expected Tthreshold for the threshold signal strength Pthreshold of the packets received can be computed as below:

| Power (watts) | P1 | P2 | Pthreshold |
| --- | --- | --- | --- |
| Time (s) | T1 | T2 | Tthreshold = ? |

Table 3.1 time threshold calculation

Tthreshold  = ( T2 – T1) * (Pthreshold - P2)  (P2 – P1)+T1                                        (1)

Routing protocol needs a time to setup an alternate path, for this purpose a time changing rate is introduced and calculated like “Tthreshold - T”, this changing rate of time should be sufficient enough to send Request message to the neighboring nodes for finding another route to the destination much earlier, for initiate the process of discovering in advance, the time changing rate should be compared with a fixed time threshold, fixed_Tth  which we introduced, so the time changing rate should be smaller than fixed_Tth to start the advance discovering route and broadcast a Request massage. The Figure 3.1 shows our proposed method.

N2 N1 N2 PThreshold TThreshold P2, T2 P1 , T1 T2 - T1 N2 TThreshold -T2

Figure 3.1 Proposed Method

Modified RREP packet format

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |
| Type |  |  |  |  |  |  |  | R | A | Reserved |  |  |  |  |  |  |  |  | Prefix Sz |  |  |  |  | Hop Count |  |  |  |  |  |  |  |
| Destination IP Address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Destination Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Originator IP Address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lifetime |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MY RSSI (Received Signal Strength Indicator ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GetTime |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 3.2 Modified RREP packet format of AODV protocol

A node may offer connectivity information by broadcasting local Hello messages [1], it should only use hello messages if it is part of an active route. Every HELLO_INTERVAL milliseconds [6], the node checks whether it has sent a broadcast message within the last HELLO_INTERVAL. A node may determine connectivity by listening for packets from its set of neighbors. If does not receive any packets (Hello messages or otherwise), it should assume that the link to this neighbor is currently lost. When this happens, the node should sends a RERR message [6].

Hello packet format has the same format of the RREP message (Figure 3.1), so the same fields of this last, it is possible to modify the existing packet format by adding additional fields:

- The first additional field named “MY RSSI” (Received Signal Strength Indicator) added for getting the received power which taken from the wireless physique layer.

- The second field is “GetTime”, is the current time when the hello packet received, it’s also added from wireless physique layer.

Modified RREQ packet format

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |
| Type |  |  |  |  |  |  |  | J | R | G | D | U | Reserved |  |  |  |  |  |  |  |  |  |  | Hop Count |  |  |  |  |  |  |  |
| RREQ ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Destination IP Address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Destination Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Originator IP Address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Originator Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Destination IP Address of NodeWllBk (Node Will Break) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 3.3 Modified RREQ packet format of AODV protocol

A node disseminates a RREQ message when it determines that it needs a route to the destination and does not have one available. This can happen if the destination is previously unknown to the node, or if a previously valid route to the destination expires or is marked as invalid [6].

In RREQ packet format we have added an additional field “Destination IP Address of NodeWllBk”, this field is filled by the destination IP address of a node which will break from an active route. It’s used when a node predict a link breakage for broadcasting another RREQ message and avoiding this destination IP address.

Each time a Hello packet is received, the receiving node monitors the link with the following algorithm:

Algorithm : Link Failure Prediction

For each neighbor when Hello packet received Get P received signal strength “RSS” & T received time. Input: list of neighbors & their received time & power = Null, Pthreshold = 3.65*10-10, fixed_Tth = 0.4s Output: a Fresh path is found in advance, before link will break 1: procedure lk_fail_predict(P2, T2, Nb_addr) where P2 - power, T2 - time, Nb_ addr neighbor@ 2: n ⟵ try_to_find(Nb_addr) 3: if n is NULL then 4: insert(Nb_addr, P2, T2) 5: else 6: if P1 > P2 then 7: Tthreshold = (( T2 – T1) * (Pthreshold – P2) / (P2 – P1)) + T1 8: if Tthreshold – T2 < fixed_Tth 9: deleteFromRt(Nb_addr) 10: else 11: P1 = P2, T1 = T2 12: else 13: P1 = P2, T1 = T2 14: end procedure 15: procedure deleteFromRt(address_BK) 16: Rt ⟵ routing table head 17: while Rt ≠ NULL do 18: if nexthop@ = address_BK then Puts the route under repair 19: SendRequest2(RtAddress_dst, address_BK) start discovering a route much earlier 20: break 21: else 22: Rt ⟵ the next 23: end while 24: end procedure

Continued

Algorithm : Link Failure Prediction

25: procedure SendRequest2(RtAddress_dst, address_BK) discovering another route to the destination address “RtAddress_dst” filling up the added field " Destination IP Address of NodeWllBk " by the address of the node which will break 26: end procedure 27: procedure try_to_find(Nb_addr) search in the “neighbors list & their received time & power” the Nb_addr if it exist, return it else return null 28: end procedure 29: procedure insert(Nb_addr, P2, T2) Fill up the “neighbors list & their received time & power” by address of neighbor “Nb_addr”, received power P2, and received power instant T2. 30: end procedure For each neighbor when RREQ (Request) packet received 30: procedure recvRequest(Packet *p) Get address of neighbor which will break, and compare it with the index address “source address” If it’s equal to the source address then packet drop. 31: end procedure

As shown in algorithm above, the implementation of our work includes six procedures:

- lk_fail_predict (P2, T2, Nb_addr)

- deleteFromRt (address_BK)

- SendRequest2 (RtAddress_dst, address_BK)

- recvRequest (Packet *p)

- try_to_find (Nb_addr)

- insert (Nb_addr, P2, T2)

First one, lk_fail_predict (P2, T2, Nb_addr) this is the main function which initiate the process of link failure prediction, it’s based on three functions, so like we talked in algorithm above for each neighbor, when Hello packet received (recvHello (Packet *p)) from the Nb_addr, it got the received power value P2, and the instant T2 of received this last, and called the function lk_fail_predict (P2, T2, Nb_addr), to start the prediction.

When the lk_fail_predict (P2, T2, Nb_addr) is called, the prediction process start from this function, the main operations made are:

- Calling try_to_find (Nb_addr) function

- Calling insert (Nb_addr, P2, T2) function if needed

- Calculating Tthreshold

- Calculating time changing rate (Tthreshold – T2)

- Calling deleteFromRt (address_BK) function

Second main function deleteFromRt (address_BK), it’s Responsible on routing management and routing table in our method. This function works by avoiding the address of the next hop from routing table, this address marked as an address of the node which will break, so the function compare the address of the node which will break with next hop address in routing table, if they are equals to each other, the function SendRequest2 (RtAddress_dst, address_BK) will call.

SendRequest2 (RtAddress_dst, address_BK) function, is the same function defined for standard AODV protocol, the only difference is that SendRequest2 filling up the added field in RREQ packet format by the destination address of the node will break “address_BK”, SendRequest2 allows to discover another route to the destination RtAddress_dst such as the next hop of this last is predicted like a node will break. The aim of SendRequest2 function is discover a route to the destination without the node which will break participating or make a part of an active route.

When a RREQ message is received, the receiving node gets the address of the node will break, from the field of the RREQ packet, then it comparing with his address, and if it has the same address, it will drop the received packet.

For try_to_find (Nb_addr), function is just for search in neighboring list and return a pointer to this node if found or return Null. For insert (Nb_addr, P2, T2) if after the search of neighbor node by the previous function and the pointer pointes to null, a node will insert in neighboring list with its received power and received instant for this power.

Conclusion

The major issue created due to nodes mobility in the Ad hoc network is Link Failure, this causes less routing performance in several metrics, the standard Ad hoc on Demand Distance Victor (AODV) routing protocol doesn’t provide a method for predicting a link breakage before a path will fail, it can only repair the route after it broken which decreases the routing performance. For this purpose, in this chapter we have proposed a link failure prediction algorithm added to AODV protocol, this method can increase the routing performance, so the next chapter proves that there is an improvement after the simulation results.