The structural description

CHAPTER

# 3

ـــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــ

The structural description:

(3-1): Introduction.

(3-2): The loads that affect the building.

(3-3): Structural elements, constituents the building.

(3-4): The expansibility spacers.

(3-1) Introduction:

The main aim of the process design facilities is to ensure the existence of the advantages of operating where necessary, to contain structural elements with dimensions on the most suitable in terms of safety and economic terms.

The knowledge of constituents elements of any construction project is essential in the design of reinforced concrete structures, in order to make comparisons between different types of these elements for the structural system safer.

Therefore, we will describe structural elements that go into the design of this project.

- The goal of structural design:

The goal of the process of structural design is the choice of an integrated system of structural and balanced, and able to afford the forces situated on it, so that the building meets the requirements and desires of the users, and thus structural elements are determined based on the following:

- The factor of safety is achieved by selecting sections for structural elements capable of withstanding the forces and stresses resulting.

- The economic cost is achieved by choosing appropriate building materials and by choosing a perfect low-cost segment.

- The building operation Validity limits (serviceability): in terms of avoiding deflections and cracks which makes the users uncomfortable.

- Maintain the architectural design.

- Maintain the surrounding environment.

- Structural design stages:

We will distribute structural design for this project in two phases:

- The first stage:  at this stage we will determine the appropriate structural system for the project and do the analysis structural of this system.

- The second stage: We do the Structural design for each element of the construction with details, accurate in accordance with the chosen construction work and construction drawings executable.

(3-2) the loads that affect the building:

It's a collection of loads which designed for the capacity of the building, And any building is exposed to several types of loads must be calculated and determined with high accuracy, because any error in identifying and calculating the loads will reflect negatively on the structural design of the various elements of construction.

The building is exposed to different loads, it determined accurately by using different codes.

- The permanent loads (dead loads):

(Figure 3-1): A picture showing the permanent loads in the buildings.

It is a permanent forces resulting from the forces of gravity, which are fixed in terms of the amount and the location and does not change during the life of the building, Those loads are represented in the weight of structural elements and weights of elements based upon a permanent basis, such as circuit breakers and walls, In addition to the weight of any object Contiguous to the building permanently and the process of calculating and estimating the loads through knowledge of the dimensions of the structural elements and specific density of the materials which used in the manufacturing process of structural elements, it often include the: concrete , reinforcing steel, plastering, bricks, tiles , finishing materials, stones used to cover the outside of the building, and there is also a pipe extensions in addition to the suspended ceilings and trims of the building.

Table (3-1): specific density of the material used

| Specific Weight KN/m3 | Material | No. |
| --- | --- | --- |
| 23 | Tile | 1 |
| 17 | Sand | 2 |
| 25 | Reinforced Concrete | 3 |
| 10 | Hollow Block | 4 |
| 22 | Plaster | 5 |
| 22 | Mortar | 6 |
| 0.1 | kalkal | 7 |

(B) Variable loads (Live loads):

It's the loads that change in terms of value, location, places and time and changing by using the building, and these loads can be there from time to time that means it can be or not, depending on the nature of the building, all of these loads contain the people, furniture, appliances, equipment and materials stored and other and these loads Can be determined by knowing the type and nature of using the building and tables prepared for this purpose in different codes.

Table (3-2): live loads of building elements

| NO. | Type of Area | Live Loads (KN/m2) |
| --- | --- | --- |
| 1 | Lecture halls | 5 |
| 2 | Roof (including snow loads) | 2 |
| 3 | Cafeteria | 5 |
| 4 | Stairs | 5 |
| 5 | Corridors | 4 |
| 6 | Laboratories | 3 |
| 7 | Ateliers | 5 |
| 8 | Offices | 2 |
| 9 | Work Shops | 5 |
| 10 | Elevator | 10 |

(C) Wind loads:

(Figure 3-2) wind loads on the building.

Wind loads affect horizontal forces on the building, the process of determining wind loads are depending on the wind maximum speed and it change with the building height above the ground and with regard to the location of the building and the other high buildings around it  or the presence of the same building in high or low site, and many other variables will be obtained on the values ​​of horizontal wind and it shows in the following equation:

Vz =V.S1.S2.S3

So that:

Q: dynamic pressure of the wind on the high specific surface level of the surrounding land and the unit (N/m2).

Vz: design speed of the wind, a wind speed at an altitude specified that the building should be designed to resist or origin and unity (m / s).

S1:   coefficient of topography determines as stated in the table.

S3: a statistical coefficient identifies as stated in the table.

S2: coefficient of rugged land and determined by what is stated in the table.

S1: 1.0          S2:0.96     S3: 1.0

V: 35 (m/s) ………4/5/3-b

Vz = 35*1.0*0.96* 1.0 =33.6 (m/s)

Q= 0.613*(33.6)2 =692.05 N/m2 = 0.692 KN/m2

And will be relying on this value of dynamic pressure of the wind to get the forces design to know the winds actions.

(D) Loads of snow:

(Figure 3-3): a normal image shows the snow loads on buildings

Snow loads can be calculated by knowing the height above the sea level and using the table below.

Table (3-3): loads of snow by the height above the sea level

| the height above the surface of the sea m | Snow loads kN /m² |
| --- | --- |
| h < 250 | 0 |
| 500 > h > 250 | 1000 / (h-250) |
| 1500 > h > 500 | (h-400) / 400 |
| 2500 > h > 1500 | (h – 812.5)/ 250 |

Based on the previous table of the snow loads and after determining building height above sea level, which is equal to (1001 m) snow loads were calculated as follows:

SL = (h-400) / 400

SL = (1001 – 400) / 400 = 1.5 KN/m²

(E) Earthquake loads:

Earthquakes produced from horizontal and vertical vibrations due to the relative movement of the layers of rocky ground and that produce shearing forces which affecting the building, and this loads must be taken into account when we do the designs so as to ensure the building resistant to earthquakes.

And it will be resistances on this project by distribute shear walls in the building based on accounts of the construction.

(3-3): Structural elements, constituents the building:

All buildings usually consist of a collection of structural elements that come together in order to maintain the continuity of the existence of the building and suitability for human use, and the most important elements of these are the  nodes, bridges, columns and load-bearing walls and so on.

- Nodes :

It's a Structural element which capable of transmitting forces anchored because of the loads affecting to the load-bearing structural elements of the building such as bridges, walls and columns, without exposure to defects.

In this project, there is  two types of nodes everyone  in the right place for it, which will be explain in structural design in the next chapter and there is a statement for these types in the following:

- One way ribbed slab.

- Solid nodes (One way solid slab).

Form (3-4) shows the shape of the node nerves.

Form (3-5): shows the solid form of a node in one way.

Form (3-6) shows the shape of the node solid two-way.

- Stairs:

It's an architectural element which used for vertical transmission between the different levels of levels through the building, and will be the design of one type of stairs structurally.

Form (3-7) shows the shape of the stairs.

- Bridges:

It is an essential structural elements in the movement of loads from the tiles to the columns, which are two types: Bridges enchanted - hidden inside nodes - and fallen bridges "Dropped beam" They that stand out from the node to the bottom. This project includes different types of bridges:

- The enchanted bridges (hidden).

- Fallen bridges (Dropped beam).

Form (3-8) shows the shape of the concrete bridge.

- Columns:

Columns are the main component in the transfer the loads from bridges to the foundations, there for it is a necessary element for the transfer of loads and the stability of the building, therefore it must be designed to be capable of carrying and distributing loads located on it, and was chosen two syllables rectangular and circular concrete columns.

Form (3-9): clip shows the column.

- Shear walls:

It is structural elements carrier resist the vertical forces which located on it and it is used primarily to resist horizontal loads such as strong winds and earthquakes, called shear walls, but it is in this project are only to resist vertical loads, the load-bearing walls in the building will be the walls of the elevators, the walls of the staircase and it is working to carry the vertical weights which transmitted to it. The building has a number of shear walls ongoing from the foundation and other which carried on the same node, and extends in both cases to the upper floors and the walls are in the staircase and elevators.

Form (3-10): shows a section of resistance wall to shear forces.

- Foundations:

Although the foundation is the first implementation will begin when construction of the building, but the design is after the completion of the design of all structural elements of the building. It’s the structural elements which distribute all loads and forces from the walls and columns to the soil, has been the adoption of the durability of the soil (5.0) kg /cm²   for the project area, and the foundations of several different types.

We will use isolated footing, combined footing and Strip footing.

Form (3-11) shows a form of a solo basis.

- The retaining walls:

Because of the presence of underground parking we had to use the retaining walls to protect the soil from sliding or collapse. The retaining walls can be implemented of reinforced concrete or regular or stone.

(3-4): The expansibility spacers:

Spacers are placed in blocks of buildings with large horizontal dimensions and shapes and special situations and those spacers: thermal expansion spacers or downs spacers. Spacers may be together for two purposes.

When analyzing the constructions for study as a resistance of acts of the earthquakes, we call it seismic spacers.

These spacers have some requirements and special recommendations according the following:

- We should use thermal expansion spacers in blocks of the building by the approved code as long as the spacers can reach to the upper face of the foundation without Breached.

- The width of the spacers must be at least 3 cm