DESIGN OF STRUCTURAL MEMBERS

CHAPTER

# 4

ـــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــــ

(4-1): Introduction.

(4-2): Design of Rib (NO.2).

(4-3): Design of beam (NO.50).

(4-4): Design of column (NO.32).

(4-5): Design of isolated footing (NO.32).

(4-6): Design of stair.

CHAPTER FOUR

Structural Analysis and Design

(4-1): Introduction:

The strength of a structure depends on the strength of the materials from which it is made. For this purpose, design material strengths are specified in standardized ways.

Actual material strengths can’t be known precisely. Structural strength depends, on the care with which a structure is built, which in turn reflects the quality of supervision and inspection. Members sizes may differ from specified dimensions, reinforcement may be out of position, poorly placed concrete may show voids, etc, and this can reduce the strength of the structure.

The design strength provided by a member, its connections to other members, and its cross-sections in terms of flexure, load, shear, and torsion is taken as the nominal strength calculated in accordance with the requirements.

So in this chapter we will explain the design of the structural element for this project, the dead load(permanent loads ) is calculated based  on type of used materials, but the live load(Variable loads) is chosen based on the values that are used in chapter three tables (2-3).

(4-2):  Design of Rib (NO. 2) in third floor:

The main loads acting on the structure are dead and live load. Dead Load is calculated based on the density for each material used in the slab.

The overall depth of slab must satisfy the limitation of deflection required.

Min h = L / 21                         for interior span

Min h = 5.4 / 21       = 26 cm

Min h = L / 18.5                 for exterior span

Min h =4.2 / 18.5       = 23 cm

Min h = L / 16               for simply support

Min h =3.8/ 16       = 24 cm

(4-2-1): Calculate the dead loads (the permanent loads):

Figure (4-1) shows a section in one way ribbed slab.

Coarse Sand                            0.07*0.55*17 = 0.655

Tile                                          0.03*0.55*22 =0.363 KN/m of rib

Concrete Rib                          0.24*0.15*25 = 0.9 KN/m of rib

Block                                       0.24*0.4*10     = 0.96 KN/m of rib

Topping                                  0.08*0.55*25 = 1.1 KN/ m

Plaster                                    0.03*0.55*22 = 0.363 KN/m of rib

Nominal Total Dead Load (permanent loads) = 4.58 KN/m of rib

Factored Total Dead Load = 1.2*4.58= 5.5 KN/m

Live load (Variable loads) = 5 KN/m2

Factored live load   =5*1.6*0.55= 4.4 KN /m

(4-2-2): Rib Design (R2):

The length of spans is shown in figure (4-2) below. And the locations of Rib1 are shown in figure (4-3).

A

A

1

1

A

A

2

2

A

A

3

3

A

A

4

5

4

0.8

3.7

0.8

0.8

4.2

0.8

0.8

5.3

0.8

0.8

4.1

0.8

4.5

5.

6.1

4.9

15.

32.

8.

55.

A

A

Figure (4-2) spans length

Figure (4-3) shows Rib location

- We get the following moment values for positive moment:

Wu = D1 + L1 = 5.5 +4.4 =9.9 KN /m

(4-2-2-1): Design for appositive moment:

Effective Flange width ():

For T- section is the smallest of the following:

=  L / 4  = 520 / 4 = 130 cm

=  + 16 t  = 15  + 16 (8) = 143 cm

= C/C  = 55 cm………………..    Control

Determine whether the rib will act as rectangular or T – section:

For   a = t   = 8 cm

Moments:   spans 1 to 4

6.4

-22.6

-13.4

-13.

-27.5

-16.6

-17.8

-31.2

-20.3

-19.8

6.7

17.2

12.5

20.5

18.6

1.53

1.55

1.84

2.06

0.66

0.84

0.83

0.85

1.

0.91

2.03

2.47

2.5

2.5

3.05

3.05

2.94

1.96

Span 4

Span 3

Span 2

Span 1

Support 3

Support 1

Support 2

#### Figure (4-4) the design moment for the Rib (R 2)

###### Mumax = 20.5 kN.m. For all spans   ……….From Figure (4-4)

Mnf = 0.85 ** tf **(d-tf/2) = 0.85 (24) (0.08) (0.55) (0.286-0.08/2)*1000

###### =220.8 kN.m

ØMnf =0.9 *220.8 =198.7 KN.m

ØMn available = 198.7 kN.m   > Mn required =20.5 kN.m

Design as a rectangular with = 55 cm

Design for span (1):

Mu = 17.2kN.m    ……………….. From Figure (4-4)

A = 0.00104*(550)* (286) = 163.6 mm

Max & Min Reinforcement of Ribs:

A=

A= 145.8 mm2

A= 127.6 mm

We use   A= 163.6 mm2

Use 2Ø12    A = 226 mm2

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(226) x 412 = 0.85 x 24 x 550 x a

a=8.3mm

C= a /0.85 = 8.3/.85=9.76

E s = (d- C) (0.003)/C

E s = (286-9.76)*(0.003) / (9.76) = 0.085

0.085 > 0.005

Ok

Design of span (2):

Mu = 12.5kN.m    ……………….. From Figure (4-4)

A = 0.00076*(550)* (286) = 118.9 mm

Max & Min Reinforcement of Ribs:

A=

A= 145.8 mm2

A= 127.6 mm

We use   A= 127.6 mm2

Use 2Ø12    A = 226 mm2

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(226) x 412 = 0.85 x 24 x 550 x a

a=8.3mm

C= a /0.85 = 8.3/.85=9.76

E s = (d- C) (0.003)/C

E s = (286-9.76)*(0.003) / (9.76) = 0.085

0.085 > 0.005

Ok

Design for span (3):

Mu = 20.5kN.m    ……………….. From Figure (4-4)

A = 0.00125*(550)* (286) = 197.2 mm

Max & Min Reinforcement 0f Ribs:

A=

A = 145.8 mm2

A = 127.6 mm

We use   A = 197.2mm2

Use 2Ø12    A  = 226 mm2

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(226) x 412 = 0.85 x 24 x 550 x a

a=8.3mm

C= a /0.85 = 8.3/.85=9.76

E s = (d- C) (0.003)/C

E s = (286-9.76)*(0.003) / (9.76) = 0.085

0.085 > 0.005

Ok

Design for span (4):

Mu = 18.6kN.m    ……………….. From Figure (4-4)

A = 0.00113*(550)* (286) = 177.26 mm

Max & Min Reinforcement 0f Ribs:

A=

A= 145.8 mm2

A= 127.6 mm

We use   A= 177.26 mm2

Use 2Ø12    A = 226 mm2

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(226) x 412 = 0.85 x 24 x 550 x a

a=8.3mm

C= a /0.85 = 8.3/.85=9.76

E s = (d- C) (0.003)/C

E s = (286-9.76)*(0.003) / (9.76) = 0.085

0.085 > 0.005

Ok

## (4.2.2.2)  Design for Negative Moment:

Support (1)

###### Mu = 13.4 kN.m      ………………… From Figure (4-4)

A = 0.00303*(150)* (286) = 130.36 mm

A=

A= 145.8 mm2

A= 127.6 mm

1.3 *130.36 =169.5 mm2              is Control

Use   2 Φ 12 mm, A = 226 mm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(226) x 412 = 0.85 x 24 x 550 x a

a=8.3mm

C= a /0.85 = 8.3/.85=9.76

E s = (d- C) (0.003)/C               E s = (286-9.76)*(0.003) / (9.76) = 0.085   0.085 > 0.005 Ok

Support (2)

###### Mu = 17.8kN.m      ………………… From Figure (4-4)

A = 0.0041*(150)* (286) = 175.06 mm

Use 2 Φ 12 mm, A = 226 mm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(226) x 412 = 0.85 x 24 x 150 x a

a=30.43 mm

X= a /0.85 = 30.43/.85=35.798

E s = (d- X) (0.003)/X

E s = (286-35.798)*(0.003) / (35.798) = 0.021

0.021 > 0.005

Ok

Support (3)

###### Mu = 20...3 kN.m      ………………… From Figure (4-4)

A = 0.00469*(150)* (286) = 201.2 mm

Use   2 Φ 12 mm, A = 226 mm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(226) x 412 = 0.85 x 24 x 150 x a

a=30.43 mm

X= a /0.85 = 30.43/.85=35.798

E s = (d- X) (0.003)/X

E s = (286-35.798)*(0.003) / (35.798) = 0.021

0.021 > 0.005

Ok

(4-2-2-3) Design of shear reinforcement:

Shear

14.2

21.1

25.4

25.5

-22.3

-22.3

-26.7

-14.9

17.9

-26.

24.9

-26.

29.2

-30.4

29.2

-18.6

#### Figure (4-5) the design Shear for the Rib (R 2)

Vu max = 26.7 KN at the face of interior support (3)…. from figure (4-5).

Φ Vc = 26.3*1.1=28.93 > Vu max =26.7

No shear reinforcement is required. According to category 2 for joist construction

Use Φ8 @ 20 cm.

To determine the distance between bars:

Avsmin≥13bwfy   ≥116fcfy

Avsmin         ≥1.2*10-4                                                Is Control

≥1.1*10-4

We Try 2∅10

2*79*10-6S = 1.2* 10-4

S= 1.3 m

d/2= 0.286/2 =0.143 m                 is Control               we use  ∅ 10 @15 cm  c/c

2Ф12 T.B

2 Ф12 B.B

Block (24X40X20)

(4-2-3) Topping Design:

Φ 8 @ 20 cm

Both ways

Live load = 5 KN/m2

Dead load:-

Tile = 0.03*22=0.66 KN/ m.

Sand =0.07*17=1.19 KN/ m.

Topping = 0.08*25 =2 KN/ m.

morter = 0.02*22 = 0.44 KN/ m.

Figure (4-6): Topping Design

Dead load = 4.3 KN/ m.

= 1.2 (4.3) + 1.6 (5) = 13.16 KN/ m

Assume slab is fixed at support point (ribs)

= - 0.175 KN.m for 1 m wide strip

Calculate modules of rapture of concrete:

…….. For a rectangular X-section

ΦMn = 0.55 (2190)(0.00107) = 1.29 KN.m ,(Φ = 0.55   for plain concrete)

ΦMn = 1.29 kN.m > Mu = 0.175 kN.m

Minimum reinforcement is required to prevent cracks and o minimizes temperature effects:

For fy = 412 Mpa, ρ = 0.0018

ρ = 0.0018

= 144 mm/1m

Use Φ 8 @ 20 cm on center both ways

(4-3) Design of Beam - (B50):

Beam No.50

Figure (4-7) Location of Beam No. 50.

(4-3-1) Design moment of beam (50):

(4-3-1-1)   Min Reinforcement 0f Beam:

A=……….. (ACI-10.5.1)

A= 7.44  6.52 cm

A= 6.52 cm

(4-3-1-2)   Positive moment reinforcement:

Moments:   spans 1 to 5

-84.6

24.

-54.7

-65.6

-283.7

-222.8

-238.8

-420.7

-351.4

-353.3

-316.9

-269.

-257.5

25.3

29.

64.7

229.3

259.7

112.3

0.83

3.03

1.45

2.8

2.55

1.77

3.54

1.11

1.15

1.42

1.45

1.34

1.23

1.05

0.45

2.3

2.8

4.05

4.05

4.35

4.35

3.24

2.16

Span 3 1

Span 2 1

Span 4 1

Support 4

Support 3

Support 2 1

Span 1 1

Support 1

Figure (4-8) the design moment for the beam (B 50)

Span (1):

###### Mu = 64.7 kN.m  …………… from Figure (4-6).

A = 0.00299*(800)* (274) =656.145mm2= 6.56 cm  > A

Use 6 Φ 12 mm, A = 6.79 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(679) x 412 = 0.85 x 24 x 800 x a

a=17.14mm

X= a /0.85 = 17.14/.85=20.16

E s = (d- X) (0.003)/X

E s = (274-20.16)*(0.003) / (20.16) = 0.0378

0.0378 > 0.005

Ok

Span (2):

###### Mu = 229.3 kN.m ……………… from Figure (4-6).

A = 0.01167*(800)* (274) =2558.33mm2= 25.58 cm  > A

Use   10 Φ 18 mm, A = 25.4 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(2540) x 412 = 0.85 x 24 x 800 x a

a=64.12mm

X= a /0.85 = 64.12/.85=75.44 mm

E s = ( d- X ) (0.003)/X

E s = (274-75.44)*(0.003) / (75.44) = 0.0079

0.00079 > 0.005

Ok

Span (3):

###### Mu = 259.7 kN.m ……………… from Figure (4-6).

A = 0.0155*(800)* (274) =3406.8mm2= 34.068 cm  > A

Use   9 Φ 22 mm, A = 34.2 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(3420) x 412 = 0.85 x 24 x 800 x a

a=86.34mm

X= a /0.85 = 64.12/.85=101.6 mm

E s = (d- X) (0.003)/X

E s = (274-101.6)*(0.003) / (101.6) = 0.0051

0.00051 > 0.005

Ok

Span (4):

###### Mu = 112.3 kN.m ……………… from Figure (4-6).

A = 0.0053*(800)* (274) =1167.85mm2= 11.678 cm  > A

Use   10 Φ 12 mm, A = 11.3 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(1130) x 412 = 0.85 x 24 x 800 x a

a=28.5mm

X= a /0.85 = 28.5/.85=33.5 mm

E s = (d- X) (0.003)/X

E s = (274-33.5)*(0.003) / (33.5) = 0.022

0.022 > 0.005

Ok

(4-3-1-3)   Negative moment reinforcement:

Support (A)

###### Mu = 65.6 kN.m ……………… from Figure (4-6).

A = 0.0031*(800)* (274) =666.108 mm2= 6.66 cm  > A

Use   6 Φ 12 mm, A = 6.79 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(679) x 412 = 0.85 x 24 x 800 x a

a=17.14mm

X= a /0.85 = 17.14/.85=20.166

E s = (d- X) (0.003)/X

E s = (274-20.166)*(0.003) / (20.166) = 0.0377

0.0377 > 0.005

Ok

Support (B)

###### Mu = 238.8 kN.m ……………… from Figure (4-6).

A = 0.0122*(800)* (274) =2681.58 mm2= 26.82 cm  > A

Use   11 Φ 18 mm, A = 27.94 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(2794) x 412 = 0.85 x 24 x 800 x a

a=70.5mm

X= a /0.85 = 70.5/.85=82.9

E s = (d- X) (0.003)/X

E s = (274-82.9)*(0.003) / (82.9) = 0.0069

0.0069 > 0.005

Ok

Support (C)

###### Mu = 353.3 kN.m ……………… from Figure (4-6).

A = 0.0198*(800)* (274) =4348.07 mm2= 43.48 cm  > A

Use   10 Φ 24 mm, A = 45.2 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(4520) x 412 = 0.85 x 24 x 800 x a

a=114.1mm

X= a /0.85 = 114.1/.85=134.2

E s = (d- X) (0.003)/X

E s = (274-134.2)*(0.003) / (134.2) = 0.003

0.003 < 0.005

Not Ok       redesign

∅=0.65+ϵs-0.0022503                                             ϵs<0.005

= 0.65 + (0.003-0.002)2503

= 0.73

A = 0.0268*(800)* (274) =5874.6 mm2> A

Use 10 Φ 28 mm, A = 61.6 cm

Support (D)

###### Mu = 269 kN.m ……………… from Figure (4-6).

A = 0.014*(800)* (274) =3086.34 mm2= 30.86 cm  > A

Use   10 Φ 20 mm, A = 31.4 cm

Check for Yielding:

T=C

As x Fy = 0.85 x fc' x a x b

(3140) x 412 = 0.85 x 24 x 800 x a

a=79.3mm

X= a /0.85 = 79.3/.85=93.3

E s = (d- X) (0.003)/X

E s = (274-93.3)*(0.003) / (93.3) = 0.0058

0.0058 < 0.005

Ok

(4-3-2) Design of Shear Reinforcement:

Shear

111.2

235.8

269.3

185.1

-82.2

-170.

-261.8

-231.

-94.8

54.7

-33.9

-95.1

124.1

-182.9

251.4

-277.4

285.

-243.9

197.9

-107.7

Figure (4-9) the design Shear for the beam B (50)

Vu max = 285 kN.  ……………from Figure (4-9).

At the edge of the support (1.5 m from the support):

We take the magnitude of the shear at the displacement = (a/2 +d)

Such that:-

a: width of support in direction of the beam .

d: effective depth of the beam.

So (a/2 +d) = (50/2+27.4) =52.4cm.

Vu = 252.1 kN.

Category (3):

ΦVc <Vu≤ ΦVc +ΦVs

Is Control

- ΦVc + min ΦVs =201.33 KN

Vu = 252.1 > ΦVc +ΦVs = 201.33

So we cannot solve in the Category (3).

Category (4):

ΦVc+ ΦVs <Vu≤ ΦVc + ∅3fc*b*d

∅3fc*b*d = 268.46 KN

ΦVc + ∅3fc*b*d  = 402.7 KN

(ΦVc+ ΦVs)=201.33 <Vu=252.1≤ (ΦVc + ∅3fc*b*d)=402.7

So we Can solve as it Category (4).

Vs = (Vu / Φ)-Vc = 157.1 KN

Select 2 Φ 10 stirrups with two legs , Av = 158 mm2

S = 10 cm

S = 27.4/ 2 = 13.7

Select   S = 10 cm.                                       Use   1Φ 10 stirrups with 2 legs @ 10 cm

At the middle of the beam (length =5.2m):

Vu =175.15 KN

Category 3:

ΦVc <Vu≤ ΦVc +ΦVs

is Control

ΦVc + min ΦVs =201.33 KN

ΦVc=134.23 <Vu=175.15≤ (ΦVc +ΦVs)=201.33

So we Can solve as it Category (3).

Avs≥13*bwfyt   =0.00065                  Is Control

Avs≥116*fcfyt*b  =0.00059

S= 2*79*10-6 /0.00065 = 0.243 m

S< 27.4/2 =13.7 cm

S< 60 cm

Use S =15 cm

Use ∅10@15 cm   c/c

(4-4) Design of column:

(4-4-1) Design of Column (C32):

The Column is an internal one.

Pu = 2100 KN

Pn req = 2100/0.65= 3230.8 KN

Use  =g =1 %

Pn = 0.8 Ag {0.85  + (1 –g) + fy * g}

3230.8 = 0.8 * Ag {0.85(24) + (1 – 0.01) + 0.01 * 412}

Ag = 0.166 m2

Select (0.3 * 0.6) m

Design of reinforcement

Ag = 0.18 m2

Pn = 0.8 Ag {0.85  + (1 –g) + fy * g}

3230.8 = 0.8 * 0.18 {0.85(24) + (1 – g) + g * 412}

g = 0.0052 < 0.01

Use g = 0.01

As req = (0.01) (60*30) = 18 cm²

Using 8Φ18 with As = 20.4 cm2 …..See to figure (4-13).

(4-4-1-1) Check slenderness effect:

…………….. ACI 10-12-2

Lu: Actual unsupported (un braced) length.

K: effective length factor (K= 1 for braced frame).

r: radius of gyration = 0.3 h =

=  = 22 ≤ 22 < 40

Slenderness will not be considered ….

Figure (4-10): cross section of column (C32)

### (4-4-1-2) Lateral Ties Selection:

For Φ 8 mm ties:

………….. Control

Use Φ8 -mm ties @ 20 cm ………. See figure (4-13)

## (4-5) Design of Isolated Footing: (F32):

Figure (4-11) Top view of Isolated Footing (F 32).

## From Column (C 32):

Factored load = 2100 KN

Assume Allowable soil pressure = 350 KN/m²

Column= 60 cm x 30 cm………. See figure (4-11)

(4-5-1) Footing Area:

Factored Load = 1500 KN

P net = 350 KN/m²

## Area (A) = Total Weight / (Soil Pressure*1.4)

= 2100 KN / 350*1.4 KN/m²                          = 4.29 m

Use L = 2. 1 m, B = 2.1 m,    A = 4.41 m………. See figure (4-11)

(4-5-2-1) Check this depth for two way shear action (punching):

The punching shear strength is the smallest of:

=

=

=…………..Control

Where:

= a / b = 60 / 30 = 2

= Perimeter of critical section taken at (d/2) from the loaded area

= 2{(0.6+0.405) + (0.3+0.405)} =3.42 m

= 40             for interior column

The depth (h =50cm) is not valid

So we select h = 60 cm………. See figure (4-15)

Check (h=60cm)

d = 60-7.5-2 = 50.5 cm

=…………..Control

= Perimeter of critical section taken at (d/2) from the loaded area

= 2{(0.6+0.505) + (0.3+0.505)} =3.82 cm

2360.3 >1676.5   KN …….OK

Figure (4-12) Side view of Footing (F 32).

(4-5-2-2) Check transfer of load at base of column:

In column:

Dowels are not required for load transfer.

But use the minimum reinforcement of dowels:

As = 0.005 *(300 x 600) = 900 mm2

Use 8 Φ 12     dowels with   = 905 mm………. See figure (4-13)

In footing:

A1 = 0.3*0.6 = 0.18 m2

A2 = 1.8 * 2.1 = 3.78 m2

Use 8 Φ 12     dowels with   = 905 mm………. See figure (4-16)

(4-5-2-3) Development Length ():

Ld for Φ 12:

=    =

==0.04*12*412 = 197.8 mm

Available embedment = 600 – 75 – (2 x 14) – 12 = 485 mm > 252 mm

OK.

(4-5-3) Design for Bending Moment:

Mu =

=  = 405 KN.m

Mn = 405/0.9 = 450   KN.m

Rn = Mn/ b.d2 = 0.84 Mpa

AS = 0.0021*2100*505 = 2227 mm2

As min = 0.0018*2100*600 = 2268 mm2          control

Use 15Φ 14           (In tow direction)

4-5-3-1 Development Length ():

Category (A), item 2 applies,

Ld for Φ 14:

=

Available embedment = (90- 10) = 80 cm > 34 cm                  ok.

Figure (4-13) Detail of Footing No. 32

(4-6) Design Double Flat Stairs:

The overall depth of solid slab of stair must satisfy the limitation of deflection

Required way solid slab:

Min h = (L /20)

L=plan length of stairs + 0.5 length of landing (or 90 cm whichever is less)

L= 0.9+3+0.85 = 4.75 m

Min h = (4.75 /20) = 0.238 m

Select h=25 cm.

Figure (4-14) Double Flats Stairs.

(4-6-1) Load Determination:

Dead load calculation of q1:            See figure (4-15)

α= tan-1(16/30) = 28

Concrete = (25*0.25)*(1/cos28) = 7.08 KN/m

Plastering = (0.03*22)*(1/cos28) = 0.75 KN/m

Stair = (0.5*0.3*0.16*25)/0.3 = 2 KN/m

morter = ((0.16+0.3)*0.02*22)/0.3 = 0.67  KN/m

Tiles = ((0.16+0.35)*0.3 = 1.38 KN/m

Figure (4-15) Section in Stair.

Nominal Total Dead Load   = 11.88 KN/m

Factored Total Dead Load = 1.2*11.88 = 14.26 KN/m

Live load = 5 KN/m.

Factored live load   =5*1.6 = 8 KN/m

q1 = 8 + 14.70 = 22.70   KN/m

Dead load calculation of q2: (for landing)

Concrete = (25*0.25) = 6.25 KN/m

Plastering = (0.03*22) = 0.66 KN/m2

Morter = 0.02*22 = 0.44 KN/m

Tiles = 0.03*22 = 0.66 KN/m2

Nominal Total Dead Load   = 8 KN/m

Factored Total Dead Load = 1.2*8 = 9.6 KN/m

Live load = 5 KN/m2.

Factored live load   =5.0*1.6 = 8.0 KN/m2

q2 = 8 + 9.42 = 17.42   KN/m2

(4-6-2) Stair reinforcement Design of one meter strip:

Mu max = 26.5 KN.m

d = h-2-1 = 250-20-7 = 223 mm

A = 0.00148*(100)* (22.3) = 3.3 cm

(4-6-2-1) Min rein forcement:

A= 0.0018 * 1000 * 250 = 4.5 cm

As min = 4.5 > 3.3 cm2

Select As= As min =4.5 cm2

As =4.5   cm2

Use Φ 12 @ 25 cm c/c   with As =4.52   cm2 > 4.5   cm2………. See figure (4-16)

Figure (4-16) details of stairs